DFF Guideform Specifications

April 09, 2002

The relay shall be an integrated digital protection system including frequency and voltage protection, voltage monitoring and communication capabilities.

Protection functions shall include:

- 8 frequency units selecting the rate of change mode or the absolute mode

- one frequency undervoltage supervision unit

- one overvoltage unit (one phase)

- one undervoltage unit (one phase)

Monitoring functions shall include:

- voltage metering

- fault voltages and frequency for last trip

- self diagnostics

- pick up and trip status of each function

- oscillography (voltages and digital flags)

- sequence of events (up to 150 internal events)

User interface shall include:

- 1 LED to indicate the relay is in service

- 16 user configurable LEDs

- 33, 25 or 13 inputs (depending on the model)

- man-machine interface (MMI) shall be in the form of a 2 line by 16 character backlit LCD alpha numeric display and keypad to accommodate programming and viewing of parameters

- RS232, RS485 and fiber optic ports shall be provided for remote or local access via a personal computer