MIN Guideform Specifications

April 09, 2002

Ground protection, control, monitoring and metering shall be provided in one integrated digital package for application on grounded generators/transformers and feeders on ungrounded or Petersen coil grounded systems and is suitable for incorporation in an integrated substation control system.

Protection and control functions shall include:

- Ground TOC for grounded systems (directional or not directional)

- Ground TOC for ungrounded systems (directional)

- Ground TOC for Petersen coil grounded systems (directional)

- Ground IOC (two levels, Hiset and Loset)

- Configurable breaker failure to open

- Directional comparison scheme logic

- Time overcurrent curve characteristics: 4 pre-configured, one user configurable

- 6 outputs, four of them configurable

- Two setting tables: one pre-configured for grounded systems, and the other for Petersen coil

- Breaker failure protection

Monitoring features shall include:

- 32 event record

- Analog/digital oscillography

- Ground current metering

- KI2 counter for breaker maintenance