RRTD Guideform Specifications

April 09, 2002

The module shall be capable of monitoring up to 12 three wire shielded RTDs. Each RTD input is to be individually field programmable as to type (100P, 100N, 120N, 10C).

The communication interface shall include 3 independent RS485 ports and a fiber optic port. ModBus® RTU is to be the standard protocol.

The module shall be accompanied by supporting PC software thus allowing easy programming and monitoring. The module shall be capable of automatically communicating the RTD information to the 369 Motor Management Relay.